MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

STONINGTON WATER COMPANY

BURNTLAND POND WATERSHED

MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

prepared by

Drumlin Environmental, LLC

Portland, Maine

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

STONINGTON WATER COMPANY

BURNTLAND POND WATERSHED

MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

11 State House Station

Augusta, Maine  04333

prepared by

Drumlin Environmental, LLC

15 Franklin Street, P. O. Box 392

Portland, Maine 04112-0392

(207) 771-5546

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

STONINGTON WATER COMPANY

BURNTLAND POND WATERSHED

TABLE OF CONTENTS

Section      Title                                                                                                            Page No.

|  | EXECUTIVE SUMMARY | ii |
| --- | --- | --- |
| 1.0 | INTRODUCTION | 1 |
| 2.0 | WATER SUPPLY SOURCE | 1 |
|  | 2.1 Description | 1 |
|  | 2.2 Land Use | 3 |
| 3.0 | INVENTORY OF POTENTIAL WATER QUALITY CONCERNS | 4 |
|  | 3.1 Background | 4 |
|  | 3.2 Source Sensitivity to Land Use | 5 |
| 4.0 | # ASSESSMENT OF BURNTLAND POND | 6 |
|  | 4.1 Watershed | 6 |
|  | 4.2 Shoreland | 6 |
|  | 4.3 Intake | 6 |
| 5.0 | SWAP RANKING AND RECOMMENDATIONS | 7 |
|  | 5.1 Ranking of Susceptibility | 7 |
|  | 5.2 Recommendations | 8 |
|  | Table 1 - Burntland Pond Supply | 2 |
|  | Table 2 - Sources of SWAP Information | 5 |
|  | Table 3 - Burntland Pond Surface Water Assessment | 7 |
|  | Figure 1 - Public Water Supply and Surrounding Watershed | End |
|  | Figure 2 - Land Under Protective Ownership | End |
|  | Figure 3 - Multi-Resolution Land Characterization | End |
|  | Figure 4 - Potential Threats to Drinking Water | End |
|  | Figure 5 - Potential Erodible Soils | End |
|  | Appendix A - SWAP Assessment Guidelines | End |

# STONINGTON WATER COMPANY

# BURNTLAND POND WATERSHED

# EXECUTIVE SUMMARY

The Stonington Water Company serves approximately 600 people from Burntland Pond and a group of eight water supply production wells located near the pond.  The 288-acre surface watershed of Burntland Pond is located entirely in Stonington.  There are no significant streams that drain into the pond.  The pond is small and shallow.  The natural flushing and buffer to runoff impacts is limited.  The Water Company relies on a newly installed filtration system to remove color and increased turbidity that results from the treatment process.

Approximately 75 percent of the watershed is covered in forest growth.   The shoreland is undeveloped except for the Water Company pumping station area and limited residential encroachment along a portion of the shore.  The Water Company owns land and a conservation easement at the intake and on land located to the west.   The town has adopted 250-foot setback Shoreland Zoning with a “no building” restriction within 100 feet of the pond.  Access to the pond and intake area is unrestricted although there is a local ordinance to control public activity on the pond.   The pond’s recreational use is limited to walking, fishing and non-motor boating.

A few potential land use threats were identified during our reconnaissance of the watershed.  These threats are associated with commercial activities located along Route 15, which can contribute both point source and non-point source runoff impacts in the watershed if adequate safeguards are not followed.  The watershed area located to the north of the pond is mostly forested and undeveloped.  South of the pond, there is a limited number of nearby residential homes and a large number along Route 15 and Cemetery Road.   The Water Company has recommended that the town adopt a watershed protection ordinance to mitigate the potential threat of future growth on both the pond and well sources.   This effort needs to be successful in order to conserve the quantity and protect the quality of the local water supply.

The overall susceptibility of the Burntland Pond water supply is ranked low to moderate based on the SWAP assessment findings.  The natural forest cover, existing low-density development and protection strategies support a low ranking.   However, evidence of encroachment by residential development and nearby commercial activities indicates that future growth of similar land uses poses a threat to the pond source.  The adoption of watershed zoning and land use practices that safeguard the water resource would mitigate those factors that raise the susceptibility to a higher rank.

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

# STONINGTON WATER COMPANY

# BURNTLAND POND WATERSHED

1.0    INTRODUCTION

The operation of public water systems in Maine is governed by the federal Safe Drinking Water Act (SDWA), which was first passed in the 1970s and later amended in 1986 and 1996.   The federal government delegated authority for enforcing the SDWA to the state under the Maine Department of Human Services (DHS) Drinking Water Program (DWP).   In 1998, the Source Water Protection Section was established in the DWP to implement the Maine Public Drinking Water Source Water Assessment Program (SWAP).  The SWAP process is being conducted through a cooperative effort between the DWP staff, Stonington Water Company (Water Company) staff and Drumlin Environmental, LLC (Drumlin), the environmental consulting firm contracted through DHS to provide technical assistance for the project.

The purpose of the SWAP evaluation for the Water Company is to assess the susceptibility of its Burntland Pond drinking water supply (PWSID  91510) to potential threats of contamination.   The long-term goal is to protect the water supply source.   To do this, the SWAP process has compiled information to assist the Water Company in future planning that will help to control potential threats to the water quality of Burntland Pond.

This Report describes the SWAP for the Stonington Water Company and is organized as follows:

- Section 2.0  summarizes the physical characteristics of the water supply at Burntland Pond;

- Section 3.0  describes the variety of conditions and activities that could pose a threat to the pond water quality;

- Section 4.0  provides an assessment of the threats within the pond watershed; and,

- Section 5.0  ranks the susceptibility of the water quality at Burntland Pond and provides recommendations for future protection of water quality.

2.0   WATER SUPPLY SOURCE

2.1   Description

The watershed of Burntland Pond is located in the coastal community of Stonington, Maine.  The pond and watershed area are depicted in Figure 1.  The components and description of the Water Company source are presented below in Table 1.

# TABLE 1

# BURNTLAND POND SUPPLY

| # Category | # Description |
| --- | --- |
| Water System Name: | Stonington Water Company |
| Surface Water Source(s): | Burntland Pond |
| Water System Type: | Non-transient, community system |
| Watershed Location: | Stonington, Maine |
| Source Surface Water Area: | 23.8 Acres |
| Source Watershed Area: | 288 Acres |
| Average Depth: | 4-5 ft |
| Water Volume in Storage: | Undetermined |
| Population Served (customers): | Services: 178 year-round, plus 72 seasonal Population: Estimated at 600 |
| Type of Treatment: | Ozonation and Chlorination |
| Filtration: | Ultrafiltration and nanofiltration system in use |
| Estimated Daily Water Use: | 44,400 GPD average, 47,000 GPD peak use |

The Water Company exists as a corporation that is owned and operated through the Town of Stonington.  The Water Company provides water to the local community through a combination of groundwater and surface water sources.  The pond source was first used in the early 1900s.

In 1995, the state asked the Water Department to stop using the pond due to treatment difficulties arising from the need to remove color.   Raw water turbidity was generally below one NTU.   However, because a coagulant was needed to remove color, the coagulant caused turbidity to be increased up to 5 NTUs.   This tended to plug the existing filters so that turbidity could not be removed to an acceptable level.   In the fall of 2001, the new filtration equipment was installed and the pond supply was reactivated.

Currently, drinking water is supplied on a shared basis from the pond and a network of eight production wells.  The wells are drilled into bedrock.  The Water Department varies the use of the wells and pond depending on day-to-day operational requirements.  The use of the pond provides the Water Department with the ability to rest the wells and allow groundwater levels to recover after a period of continuous use.   The pond is used less in the summer months when the warmer temperature and quality parameters increase the cost of filtration.   On average, approximately 10 percent of the total supply to the village residents is supplied from the pond in the summer and about 30 percent is supplied from the pond in the winter.

The intake and pumping station at the pond is accessed from the south by Pumping Station Road.  The 10-inch diameter intake pipe is positioned approximately 300 feet from shore at the southwest end of the pond.  The pipe rests on the pond bottom at a depth of five feet with a vertical riser pipe equipped with drilled holes at the end.  During the course of a normal year of precipitation and water use, the pond reportedly fluctuates within a range of about two feet.  The intake is not marked with buoys or signs to indicate its location.

Water is pumped from the pond into the pump station building where it is ozonated and filtered using ultrafiltration and nanofiltration equipment.  Water from the production wells is also directed through this treatment system and may become blended with pond water depending on the cycling of usage of both sources.  Sodium hypochlorite is added on the downstream side of the filters as the water flows into a 10,000-gallon, inground storage tank to provide contact time for chlorination.  The water is subsequently pumped from the tank to a standpipe where it flows by gravity into the distribution system, which feeds the village area of Stonington.

2.2   Land Use

The Water Company owns three small parcels of land that are not contiguous.  The Water Company has also obtained a protective easement on a larger area of land to the west and south extending from the western shore of the pond.  The easement area was established to protect the wellhead areas around its production wells and a large wetland area located adjacent to the pond.   Figure 2 shows the land under permanent protection in the watershed.  The remaining area in the watershed is divided into small- to medium-sized parcels of privately owned land.

The town has adopted Shoreland Zoning (250-ft setback) that applies around the shore of the pond.  Building is not allowed within 100 feet of the pond.  The Water Company has been working with the town to establish a new Water Supply Protection Area that applies to the entire watershed of the pond.  The ordinance is intended to provide more controls on development and reduce the density of future development by increasing lot size.   The new ordinance was not accepted by a recent vote of town meeting, which was held in March 2002.  The Water Company plans to continue the effort of educating the public about the importance of adopting this ordinance so that is can be successfully adopted at a future town meeting.

Burntland Pond is located upstream and immediately to the west of Route 15, which is the main access road into the village area of Stonington.  The eastern and southern boundaries of the watershed generally coincide with Route 15 and Cemetery Road (on the south).   The pond and nearby residential areas located to the south of the pond can be routinely accessed off the pumping station road, which has a connection to the south to Cemetery Road and to the east to Route 15.   A second dirt road provides access from Route 15 near the outlet of the pond.  The dirt road is unimproved and partially overgrown, but is designated for use by the local fire department.

Most of the land around the pond is undeveloped and forested except for the southern half of the watershed.  A low density of residential housing occurs in this area with a slightly greater density of development located along Route 15 and Cemetery Road.   Several commercial uses are also present along Route 15.  The shoreland around the pond is undeveloped except for the Water Company and some limited residential encroachment located along the southeast shore of the pond.

Approximately 75 percent of the watershed is covered by evergreen, deciduous and mixed forest growth.  Wetlands comprise about seven percent of the watershed, with the largest areas located to the west and south of the pond.   The wetlands attract beaver and waterfowl to the pond and surrounding marsh areas.  The remaining area contains residential, commercial and transitional types of land uses.   Woodcutting and farming are minor and related to residential properties.  A map of the land use classifications for the watershed and surrounding area is shown in Figure 3.

Access to the pond is generally unrestricted although the town has an ordinance that controls the types of uses allowed on the pond.  The Water Company reported that recreational activity on the pond is primarily limited to walking on trails, fishing and boating without gasoline-powered engines.   All types of fuel-powered equipment and engines are prohibited on the pond in summer and winter.  Swimming, wading, hunting, etc. are not allowed.  The access road to the intake is not closed since local residents use the same road to reach their homes.  The Water Company has a sign at the pump station building, although not all use restrictions are posted.  Access into the intake area is unrestricted since no signs or gates exist to prevent transient visitors from entering the Water Company property.   Swimming and motorboats are not allowed on the pond although non-motor boats are permitted.

3.0   INVENTORY OF POTENTIAL WATER QUALITY CONCERNS

### 3.1   Background

### The Source Water Assessment Program (SWAP) is intended to assist water companies and departments in protecting the quality of their surface water supply by identifying potential threats to water quality.  Two factors have been considered in identifying and assessing the potential risk posed by a threat: (a) location in relation to the intake and (b) the nature of a potential threat.

This SWAP assessment looks for potential threats to water quality in three zones:

- The intake zone within 1,000 feet of the intake;

- The shoreline zone, within 250 feet of the shoreline of the surface water body; and,

- The watershed zone, extending to the limits of the surface watershed.

Activities in each of these zones have a different potential effect on the quality of water in the surface supply.

Within each of the three zones, the SWAP assessment examined a variety of conditions, land uses, and practices that have a potential to influence water quality.  These features generally fall into one of the following categories:

- Physical characteristics of the watershed, for example the presence of wetlands, steep topography or erodible soils;

- General land uses and development patterns, for example the percentage of developed versus undeveloped land and controls to guide future development; and,

- Specific activities that involve chemical handling or may release pathogens (e.g., fecal coliform) with the potential to degrade water quality.

The SWAP assessment also considered available water quality data from the surface supply as an indicator of existing conditions.  Appendix A describes specific guidance used during the SWAP to inventory potential factors influencing water quality.  Appendix A also lists specific activities included in the SWAP assessment.  Table 2 lists general sources of information consulted during SWAP research.

# TABLE  2

SOURCES OF SWAP INFORMATION

| Information Type | Information Source |
| --- | --- |
| Watershed Characteristics | US Geological Survey (Topography, Hydrology), Soil Conservation Service (Soils), Maine Geological Survey (Geology) |
| Raw Water Quality | Water Company Monitoring |
| Potential Contamination Sources | DEP Databases and Water Company |
| Land Use and Zoning | Water Company and Municipal Records |
| Other Analyses | Other DEP Databases |

3.2  Source Sensitivity to Land Use

The key watershed features that define the sensitivity of the water supply source to land uses are briefly described below:

- Natural Features:   The watershed is largely forested and contains wetlands that are not conducive to development for residential or other purposes.  The areas of the watershed and shallow pond are small; therefore, the pond’s storage capacity, natural flushing and buffers to development are limited.

- Man-made Features:   The land in the area south of the pond currently has a low density of residential development.  However, the remaining undeveloped land is already divided into smaller size lots and a larger number of private landowners.  These factors can make it more difficult to institute land use controls if ownership goals are not compatible with water supply protection strategies.

- Specific Activities:   Access to the pond shoreland is generally unrestricted and some encroachment of development on the shoreland has occurred.   Wildlife, specifically seagulls and beaver, frequent the pond and may need to be managed to mitigate impacts on water quality.

4.0   ASSESSMENT OF BURNTLAND POND

Burntland Pond is used as an ozonated, filtered and chlorinated water supply source.   Raw water data for the pond has not been collected over the years other than turbidity measured at one NTU or lower.  The brownish color of the water in the pond is partly attributed to high iron and manganese.  The water supply has been tested for TE2, TE3 and other parameters after treatment and found to meet applicable state and federal drinking water standards.

4.1   Watershed

The size of Burntland Pond and its watershed is relatively small.  No significant streams flow into the pond.  The depth of the pond is shallow and provides a limited buffering and storage capacity.   The watershed is mostly forested and undeveloped to the north of the pond, lightly developed with residential homes to the south, and, more continuously developed along the roadway corridors of Route 15 and Cemetery Road.  The Water Company has protected a portion of the watershed through ownership and a conservation easement, and is promoting the adoption of a watershed-wide protection ordinance to control the potential impacts of future growth.

Review of the DEP databases of regulated activities and observations made during our reconnaissance of the watershed identified several potential threats to the water supply.  These threats include commercial uses located along Route 15 and the gravel access road (e.g., petroleum storage, motor and engine repair, and wastewater disposal).  The activities connected with these sites may contribute both point source discharges and non-point source runoff impacts in the watershed, particularly if adequate management safeguards are not implemented.  Specific sites identified as potential threats are shown in Figure 4.

4.2   Shoreland

Most of the shoreland is undeveloped and has natural vegetative cover.   Residential development has encroached somewhat on the southeastern shore.  Shoreland zoning provides a “no-build” restriction within 100 feet around the shore of the pond and provides other controls in the 250-foot buffer zone.  The shoreland topography around the pond is flat.   The natural shoreland cover and flat ground are not sensitive to erosion (see Figure 5).  The Water Company is aware of no erosion problems.

4.3   Intake

The Water Company owns a small parcel of land located at the intake although access to the area is not restricted.  The intake pipe is located in the pond approximately 300 feet from shore and is not presently identified by signs or buoys.  On occasion, people enter the pond at the pumping station and intake area for boating.  However, the Water Company reported no problems since the amount of public use has been limited.

Lakes and ponds normally experience an aging process known as eutrophication, which is caused by various natural and man-made influences.   Phosphorus from runoff, fertilizers, sewage and other sources is a primary factor affecting pond eutrophication since phosphorus nourishes plant and algae growth in the pond.   The amount of algae productivity in the pond is characterized in terms of three Trophic States: (1) Oligotrophic = low, (2) Mesotrophic = moderate, and (3) Eutrophic = high.  The Trophic State can be identified based on chlorophyll and phosphorus content and by secchi disk transparency data, which measures the clarity of the pond water.  Such data are not available for Burntland Pond.  The pond exhibits low turbidity, however the water is brown and likely contains iron and manganese in addition to residues from decaying organic vegetation associated with the nearby wetlands.

According to a surface water classification database developed by the DEP, the Trophic State of Burntland Pond is listed as Eutrophic.  The pond water is ranked as fully attaining the GPA classification with respect to primary contact (i.e., swimming), aquatic life and trophic stability criteria.

5.0   SWAP RANKING AND RECOMMENDATIONS

5.1   Ranking of Susceptibility

The SWAP assessment factors indicate that overall susceptibility of the water quality in Burntland Pond is low to moderate.  The low ranking is based on the general conditions observed, including the low-density of development, natural forested cover and efforts by the Water Company to establish water resource protection throughout the watershed.  The ranking is elevated to moderate since some encroachment by development has already occurred and future growth is likely if zoning or other protection strategies are not adopted.   In addition, the physical features of the pond favor a greater susceptibility to future growth.   That is, its small size, limited storage, slow flushing, excess color and natural organic loading cause the pond to be more sensitive to changes in the environment.   Specific factors considered in assessing the overall risk are summarized in Table 3.

# TABLE  3

# BURNTLAND POND SURFACE WATER ASSESSMENT

| Zone | Measure | Findings | Risk Level |
| --- | --- | --- | --- |
| Watershed | Ambient Water Quality Existing Conditions Future Development Overall | Class GPA Land use characteristics present some threat to water quality; Activities at commercial sites need to be protective of the water supply. Additional protection strategies including zoning controls are warranted to mitigate further encroachment and greater density of development. | Low Low-Moderate Moderate Low-Moderate |
| Shoreland | Pond Classification Soils Activities Posing a Threat Potential for Future Threats Overall | Eutrophic No erosion problems; 100-foot “No Build” Shoreland Zone protection. Encroachment by development in absence of more protection. Same as above. Additional protection strategies are warranted. | High Low Low-Moderate Low-Moderate Low-Moderate |
| Intake | Raw Water Quality Ownership/Control Activities Posing a Threat Potential for Future Threats Overall | High color, iron, manganese and natural organic residue. Water Company ownership and conservation easement provide protection near intake. Public access not restricted though no problems reported. Unauthorized entry and vandalism. | Low-Moderate Low Low Low-Moderate Low |
| Overall |  |  | Low-Moderate |

5.2   Recommendations

The overall ranking for the pond susceptibility to threats of contamination is low to moderate.  Several additional actions could be considered by the Water Company and the town to provide added protection to the quality of the water supply.

- Consider implementing a water quality-monitoring program to establish the existing baseline conditions and future trends for the pond.  Baseline monitoring for nutrients and increased trophic conditions will indicate if future development is impacting pond quality.

- The Water Company should consider installing signs along nearby roadways, around the shore of the pond and/or at points of access to provide public notice of use restrictions.  The posting of water supply signs and developing public awareness for minimizing use of the pond would further protect the quality of the source.

- Shoreland and watershed areas currently have some protection under existing zoning and ownership.   The Water Company is working diligently to establish a protection zone throughout the watershed.   The findings of this SWAP assessment indicate that this ordinance is an important tool for protecting Burntland Pond as a water supply for the Stonington community.

APPENDIX A

#### SWAP ASSESSMENT GUIDELINES

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

STONINGTON WATER COMPANY

BURNTLAND POND WATERSHED

# APPENDIX A

# ASSESSMENT GUIDELINES

| Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Physical Characteristics of the Watershed Watershed boundary, area Area of water body Tributaries Watershed topography Wetlands Water body depth Sand and gravel aquifers Soil types (erodibility and slope) | smaller smaller few low many deep few low | >> >> >> moderate >> >> some moderate | larger larger many steep, rugged few shallow many high |
| Raw Water Quality Secchi Disk Transparency Turbidity pH Phosphorus Pesticides and herbicides Color Total Coliform and E Coli Dissolved Oxygen Temperature VOCs Chlorophyll-a Total Organic Carbon | > 8 M < 1 NTU 6.5-8.5 < 10 ppb ND < 5 > 29/100 ml >7 mg/L ambient absent < 2 ppb < 4 mg/L | 4-8 M < 5 NTU N/A < 20 ppb N/A 5-15 occasional 5-7 mg/L N/A 2-6 ppb 4-8 mg/L | < 4 M > 5 NTU < 6.5 or > 8.5 > 30 ppb detected > 15 > 142/100 ml < 5 mg/L > 10o C ambient detected > 6 ppb > 8 mg/L |
| Potential Contamination Source Inventory UST/AST leak sites Municipal/Comm/Special Landfill Compost sites Demolition debris sites Ash/septage storage/utilization Superfund site Sand/salt storage sites Petroleum spill sites Hazardous waste sites Wastewater treatment facilities Food residuals utilization sites Uncontrolled hazardous sites Tank farms Industrial complexes | Absent from watershed or actively monitored and controlled or fully remediated. | Present in watershed, some operational deficiencies and/or remediation is underway. | Present and unmonitored and/or unremediated. |

| Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Potential Contamination Source Inventory Non-point pollution sources Transfer stations Sludge utilization sites Automobile graveyards Engineered subsurface systems Woodyards Underground injection wells Surface impoundments Mining/mineral processing sites Overboard discharges to rivers Active UST sites Roadways Railroads Utility Rights-of-way Boat launches Commercial farms CAFOs Commercial forest operations Private septic systems Home heating oil tanks Airports Bathing beaches Combined sewage overflows | If present, at low density, properly installed and operated. In compliance with applicable local, state and federal regulations. | >>>> | High density, little evidence of active management, documented incidence of problems and failures. |
| Land Uses and Zoning Residential density Percent of watershed Owned by PWS Public lands or conservation Forested With protective zoning Agricultural Impervious cover Industrial/commercial Waterbody uses: Restricted areas Ice fishing Boating Swimming Recreational vehicles | > 2 Ac/lot total > 80% for all protected uses < 20 % total < 20 % total < 20 % total large, posted limited to absent | 1-2 Ac/lot intake and shoreland protected, others developed 20-30% total 20-30% total 20-30% total intake zone moderate, well-managed | < 1 Ac/lot critical areas (intake, shoreland) unprotected or developed > 30% total of 3 > 30% total of 3 > 30% total of 3 small to none prevalent, uncontrolled |
| Other Analyses Trophic State Index Vulnerability Index Build-out Analysis | < 25 low fully developed | 25-60 moderate >>> | > 60 high high potential for development |