MAINE PUBLIC DRINKING WATER

#### SOURCE WATER ASSESSMENT PROGRAM

KITTERY WATER DISTRICT

BOULTER, FOLLY & MIDDLE PONDS

AND BELL MARSH RESERVOIR WATERSHEDS

###### MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

prepared by

Drumlin Environmental, LLC

Portland, Maine

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

KITTERY WATER DISTRICT

##### BOULTER, FOLLY & MIDDLE PONDS

AND BELL MARSH RESERVOIR WATERSHEDS

MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

11 State House Station

Augusta, Maine  04333

prepared by

Drumlin Environmental, LLC

15 Franklin Street, P. O. Box 392

Portland, Maine 04112-0392

(207) 771-5546

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

KITTERY WATER DISTRICT

BOULTER, FOLLY & MIDDLE PONDS

AND BELL MARSH RESERVOIR WATERSHEDS

TABLE OF CONTENTS

Section      Title                                                                                                                 Page No.

|  | EXECUTIVE SUMMARY | ii |
| --- | --- | --- |
| 1.0 | INTRODUCTION | 1 |
| 2.0 | WATER SUPPLY SOURCE | 2 |
|  | 2.1 Description | 2 |
|  | 2.2 Land Use | 3 |
| 3.0 | INVENTORY OF POTENTIAL WATER QUALITY CONCERNS | 4 |
|  | 3.1 Background | 4 |
|  | 3.2 Source Sensitivity to Land Use | 5 |
| 4.0 | # ASSESSMENT OF PONDS AND RESERVOIR | 5 |
|  | 4.1 Watershed | 5 |
|  | 4.2 Shoreland | 6 |
|  | 4.3 Intake | 7 |
| 5.0 | SWAP RANKING AND RECOMMENDATIONS | 7 |
|  | 5.1 Ranking of Susceptibility | 7 |
|  | 5.2 Recommendations | 8 |
|  | Table 1 - Ponds and Reservoir Supply | 2 |
|  | Table 2 - Sources of SWAP Information | 5 |
|  | Table 3 - Kittery Water District Surface Water Assessment | 8 |
|  | Figure 1 - Public Water Supply and Surrounding Watershed | End |
|  | Figure 2 - Land Under Protective Ownership | End |
|  | Figure 3 - Multi-Resolution Land Characterization | End |
|  | Figure 4 - Potential Erodible Soils | End |
|  | Appendix A - SWAP Assessment Guidelines | End |

# KITTERY WATER DISTRICT

# BOULTER, FOLLY & MIDDLE PONDS

# AND BELL MARSH RESERVOIR WATERSHEDS

# EXECUTIVE SUMMARY

The Kittery Water District provides drinking water to more than 4,800 residential and commercial customers from three ponds and a reservoir located in northwestern York.  Sources include:

- Bell Marsh Reservoir, a 226-acre reservoir constructed in 1988 along Smelt Brook;

- Folly Pond, a 54-acre pond;

- Middle Pond, a 37-acre pond; and,

- Boulter Pond, a 102-acre pond.

Water from the Bell Marsh, Folly and Middle sources drains via a combination of pipes and overland flow into Boulter Pond, where it is withdrawn for treatment and distribution.

The Bell Marsh Reservoir has a 1,744-acre watershed located predominantly in York, with a small portion extending into South Berwick.  The 2,325-acre watershed surrounding Folly, Middle and Boulter Ponds lies entirely within York.  The Water District owns approximately 2,500 acres of land in the two watersheds.  More than 99 percent of the two watersheds is currently undeveloped.  Development of private land within the watersheds is controlled by a Watershed Protection District zone that limits development to agriculture, timber harvest or single-family development on large lots.

The reconnaissance of the watershed and shoreland areas did not identify any existing commercial or industrial activities or other specific land uses that handle or store materials that would pose a threat to water quality of the reservoir or ponds.  Access to Folly, Middle and Boulter Ponds is limited to woods trails with locked gates.  Bell Marsh Road does provide roadway access along the west side of Bell Marsh Reservoir.  There is currently only limited residential development along Bell Marsh Road.  However there is a potential for soil erosion from the gravel road surface and sparsely vegetated slopes between the road and the Reservoir, which could lead to degradation of water quality.  Erodible soils have also been mapped along several shoreline sections of Folly, Middle and Boulter Ponds, though there is no development along these shorelines to increase erosion.

There are no permitted recreational uses on any of the Water District supply ponds.  Access for recreational use is restricted by the remoteness of the ponds.  Water quality data from Boulter Pond collected in the 1970s classify the pond as mesotrophic, which suggests a moderate susceptibility, should land uses change in the future.  Water classification data have not been gathered from the other sources, however water quality entering the filtration plant has been good.

The watersheds are sparsely developed and future development is restricted by Water District ownership and zoning.  Recreational uses are not permitted on the ponds and reservoir.  Based on these factors, the overall susceptibility of the Water District water supplies is considered to be low.

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

KITTERY WATER DISTRICT

BOULTER, FOLLY & MIDDLE PONDS

AND BELL MARSH RESERVOIR WATERSHEDS

1.0   INTRODUCTION

The operation of public water systems in Maine is governed by the federal Safe Drinking Water Act (SDWA), which was first passed in the 1970s and later amended in 1986 and 1996.   The federal government delegated authority for enforcing the SDWA to the state under the Maine Department of Human Services (DHS) Drinking Water Program (DWP).   In 1998, the Source Water Protection Section was established in the DWP to implement the Maine Public Drinking Water Source Water Assessment Program (SWAP). The SWAP process is being conducted through a cooperative effort between the DWP staff, Kittery Water District staff and Drumlin Environmental, LLC (Drumlin), the environmental consulting firm contracted through DHS to provide technical assistance for the project.

The purpose of the SWAP evaluation for the Kittery Water District (Water District) is to assess the susceptibility of its drinking water supplies from Boulter Pond, Folly Pond, Middle Pond and Bell Marsh Reservoir (PWSID  90790) to potential threats of contamination.   The long-term goal is to protect the water supply source.   To do this, the SWAP process has compiled information to assist the Water District in future planning that will help to control potential threats to the water quality of its source ponds.

This Report describes the SWAP for the Kittery Water District and is organized as follows:

- Section 2.0  summarizes the physical characteristics of the water supply at Boulter Pond, Folly Pond, Middle Pond and Bell Marsh Reservoir;

- Section 3.0  describes the variety of conditions and activities that could pose a threat to water quality;

- Section 4.0  provides an assessment of the threats within the watersheds of the ponds and reservoir; and,

- Section 5.0  ranks the susceptibility of the water quality at the ponds and reservoir and provides recommendations for future protection of water quality.

2.0   WATER SUPPLY SOURCE

2.1  Description

The Water District draws its source of supply from three ponds and a constructed reservoir located in the Town of York.  Bell Marsh Reservoir was constructed in 1988.  The Bell Marsh Reservoir watershed extends north from York into the Town of South Berwick.  The Water District also draws water from Folly Pond, Middle Pond and Boulter Pond, which lie east of Bell Marsh Reservoir.  The combined watershed for the three ponds lies within the Town of York.  Under typical operating conditions, the Water District draws all of its water from its intake at the south end of Boulter Pond.  Water from Folly Pond flows overland into Middle Pond, then via a pipe to Boulter Pond.  Water from the Bell Marsh Reservoir also flows into Boulter Pond via a combination of pipes and overland flow.   The Water District does have the capability to withdraw water from the south end of Middle Pond via a pipe to its filtration plant at the south end of Boulter Pond.  The watershed areas for the ponds and reservoir are depicted in Figure 1.  The components and description of the Water District water sources are summarized in Table 1.

# TABLE  1

KITTERY WATER DISTRICT PONDS AND RESERVOIR SUPPLY

| # Category | # Description |
| --- | --- |
| Water System Name: | Kittery Water District |
| Surface Water Source: | - Bell Marsh Reservoir - Folly Pond - Middle Pond - Boulter Pond |
| Water System Type: | Non-transient, community system |
| Watershed Location: | York, Maine |
| Source Surface Water Area: | - Bell Marsh: 226 Acres - Folly: 54 Acres - Middle: 37 Acres - Boulter: 102 Acres |
| Source Watershed Area: | - Bell Marsh: 1,744 Acres - Folly, Middle and Boulter: 2,325 Acres |
| Maximum Depths | - Bell Marsh: 30+/- ft - Folly: 15+/- ft - Middle: 20 +/- ft - Boulter: 20 +/- ft |
| Reported Safe Yield | 5.6 MGD (Combined) |
| Population Served (customers): | Services: 4,800 Population: 11,625 |
| Type of Treatment: | Filtration, Chlorination |
| Filtration: | Coagulation, Flocculation, Filtration |
| Estimated Daily Water Use: | 2.7 MGD average; 3.35 MGD peak |

The Bell Marsh Reservoir is a 226-acre reservoir in the northwest corner of York.  It was constructed in the late 1980s along Smelt Brook below Bell Marsh.   The majority of the Bell Marsh watershed lies in York, with approximately 25 percent of the watershed in South Berwick.  Water from the Bell Marsh Reservoir enters a pipe at the spillway where it flows southeast toward Boulter Pond.  The pipe ends approximately 1,000 feet from Boulter Pond and the water flows overland into Boulter Pond.

Folly, Middle and Boulter Ponds are located in the northern part of York, east and southeast of the Bell Marsh Reservoir.  The three ponds lie within a watershed of approximately 2,400 acres that is located entirely in York.  Folly and Middle Ponds are separated by a narrow strip of land.  Water flows along a short stream from Folly to Middle Ponds.  Boulter Pond lies at the south end of the watershed, approximately one mile south of Middle Pond.  Typically, water flows from Middle Pond to Boulter Pond via a pipe.  A second, backup pipe allows water to be withdrawn directly from Middle Pond.

The Water District intake is located in approximately 20 feet of water immediately above the dam at the south end of Boulter Pond.  The intake structure is accessible from the dam by a metal catwalk.  Water withdrawn from Boulter Pond flows by gravity to the filtration plant located below the Boulter Pond dam where it is treated using conventional filtration and chlorinated for distribution.

2.2   Land Use

Land in the Bell Marsh Reservoir watershed and the watershed of Folly, Middle and Boulter Ponds is largely undeveloped.   The Water District owns approximately 2,500 acres of land (approximately 60%) of the land in the watersheds (see Figure 2).  The remainder of the land in the watersheds is privately owned.

The Town of York has adopted a Watershed Overlay zoning district encompassing the Water District supply watersheds.  Land use in the Watershed Overlay District is limited to agriculture, single family dwellings (10-acre minimum lot size adopted November 2000) and timber harvesting.

More than 99 percent of the watershed areas surrounding the reservoir and ponds used by the Water District is undeveloped.  The only developed land is low-density residential development, which occupies less than one percent of the watersheds.   Timber harvesting in the watershed is limited to small-scale operations conducted by private landowners.   A map of the land use classifications for the watershed and surrounding area is shown in Figure 3.

According to the Water District, all recreational uses are prohibited on its reservoir and supply ponds.  Folly, Middle and Boulter Ponds are accessible only from gravel trails with gated access.  The Bell Marsh Reservoir is bordered on the west by Bell Marsh Road, however there are no maintained access points to the reservoir from Bell Marsh Road.  A gravel trail with gated access is used by the Water District to reach the dam near the south end of the reservoir.

3.0   INVENTORY OF POTENTIAL WATER QUALITY CONCERNS

### 3.1  Background

### The Source Water Assessment Program (SWAP) is intended to assist water districts and suppliers in protecting the quality of their surface water supply by identifying potential threats to water quality.  Two factors have been considered in identifying and assessing the potential risk posed by a threat: (a) location in relation to the intake and (b) the nature of a potential threat.

This SWAP assessment looks for potential threats to water quality in three zones:

- The intake zone within 1,000 feet of the intake;

- The shoreline zone, within 250 feet of the shoreline of the surface water body; and,

- The watershed zone, extending to the limits of the surface watershed.

Activities in each of these zones have a different potential effect on the quality of water in the surface supply.

Within each of the three zones, the SWAP assessment examined a variety of conditions, land uses and practices that have a potential to influence water quality.  These features generally fall into one of the following categories:

- Physical characteristics of the watershed, for example the presence of wetlands, steep topography or erodible soils;

- General land uses and development patterns, for example the percentage of developed versus undeveloped land and controls to guide future development; and,

- Specific activities that involve chemical handling or may release pathogens (e.g., fecal coliform) with the potential to degrade water quality.

The SWAP assessment also considered available water quality data from the surface supply as an indicator of existing conditions.  Appendix A describes specific guidance used during the SWAP to inventory potential factors influencing water quality.  Appendix A also lists specific activities included in the SWAP assessment.  Table 2 lists general sources of information consulted during SWAP research.

# TABLE  2

SOURCES OF SWAP INFORMATION

| Information Type | Information Source |
| --- | --- |
| Watershed Characteristics | US Geological Survey (Topography, Hydrology), Soil Conservation Service (Soils), Maine Geological Survey (Geology) |
| Raw Water Quality | Water District Monitoring, PEARL |
| Potential Contamination Sources | DEP Databases, Water District and Municipal Contacts |
| Land Use and Zoning | Water District and Municipal Records |
| Other Analyses | Other DEP Databases |

3.2  Source Sensitivity to Land Use

The key watershed features that define the sensitivity of the water supply source to land uses are briefly described below:

- Natural Features:   The watershed is largely forested with interspersed wetland areas.  The topography in the watersheds includes areas with moderate to steep slopes as well as areas with minimal slopes.   The three ponds are relatively small, ranging in size from 37 to 102 acres.

- Man-made Features:   There is minimal development in the Water District watersheds.  The most significant feature and only uncontrolled access into the watersheds is Bell Marsh Road along the western side of Bell Marsh Reservoir.

- Specific Activities:   The undeveloped character of the watershed has limited the number of activities posing a threat to the water supply.  Potential concerns include runoff from roadway drainage and roadway-related erosion.

4.0   ASSESSMENT OF BELL MARSH RESERVOIR AND FOLLY, MIDDLE & BOULTER PONDS

The Water District reservoir and ponds are filtered and chlorinated water supply sources.   Based on available data and discussions with the Water District staff, the drinking water quality derived from these sources has historically been good.

4.1  Watershed

The Bell Marsh Reservoir is fed by several small streams, Smelt Brook and Bell Marsh.  The watershed is more than 99 percent undeveloped land.  The only current access to land in the watershed is along Bell Marsh Road.  Future development of the portion of the watershed in York is controlled by a Watershed Protection Overlay District.    Land in the South Berwick portion of the watershed is zoned to permit single- or two-family houses (80,000 ft2 minimum lot size), agriculture and timber harvesting.  Conditional uses could include junkyards, campgrounds and other development that may influence water quality.  Future development pressure in York County, including the Towns of York and South Berwick, is likely to be high.  This development pressure could include areas with less restrictive zoning in the Bell Marsh Reservoir watershed.

Folly, Middle and Boulter Ponds are fed by numerous small streams and wetland areas within the 2,325-acre watershed surrounding these ponds.  The watershed around these ponds is also undeveloped.  There are currently no public access ways into the watershed.  All access trails are restricted by locked gates maintained by the Water District.  Future development in the watershed is restricted by a combination of land ownership by the Water District and the York Watershed Protection Overlay District.

There is reported to be periodic timber harvesting on private land within the watersheds.  Timber management is practiced by private landowners, who are less likely to use herbicides than larger commercial operations.  Harvesting conducted in the watershed does, however, have a potential to disturb ground cover vegetation and increase soil erosion.  According to the Water District, the York Codes Enforcement Office provides valuable enforcement of land use ordinances and practices, including timber-harvesting activities.

Review of the DEP databases of regulated activities and observations made during an October 2000 reconnaissance of the watershed did not identify any existing point sources of potential contamination that may pose a concern to the water supply.

4.2   Shoreland

The shoreland zone around Folly, Middle and Boulter Ponds is undeveloped, as is the eastern shoreline of the Bell Marsh Reservoir.  The Water District owns the shoreline around all of its sources of supply.  Bell Marsh Road, which runs along the west shore of the Bell Marsh Reservoir, has a current and future potential to influence water quality in the reservoir.  Currently, the road has a gravel surface along much of the watershed.  Soil eroding from the gravel road surface into the reservoir will carry phosphorous, which could accelerate the natural aging (eutrophication) process and lower water quality.  According to the Water District, the Town of York plans to pave Bell Marsh Road in the future.  The town has also agreed to alter the road alignment in several locations to maintain more distance between the road and the reservoir.  In addition to potential erosion from Bell Marsh Road, steep slopes with potentially erodible soils were observed between the reservoir and Bell Marsh Road.  Vegetation cover in these areas may have been altered due to construction and filling of the reservoir, resulting in increased erosion potential until disturbed areas have revegetated.

In the future, there is a potential for increased residential development within the Bell Marsh watershed due to the access provided by Bell Marsh Road.  Current zoning restrictions should limit the density of future residential development.

In addition to erosion-related impacts along Bell Marsh Road, there are a number of areas along the shoreline of the ponds where the erodibility of native soils is high.  These areas are shown in Figure 4.  Water District personnel report that the shoreline is well stabilized with mature vegetation, and significant erosion has not been noted along the shoreline of the ponds.

4.3   Intake

While there are dams and spillway or intake structures on all of the Water District sources, water is typically withdrawn from Boulter Pond.  The Water District owns the land immediately around the intake at Boulter Pond.  The filtration plant at Boulter Pond is located below the dam.  Petroleum and other chemicals stored for use at the filtration plant does not pose a threat to the water supply.  Recreational activities are prohibited on all of the water supplies, and the Boulter Pond intake is marked by the catwalk structure from the dam.

Water is transferred from Folly and Middle Ponds and from the Bell Marsh Reservoir into Boulter Pond via pipes or overland flow.  The overland flow between Folly and Middle Ponds is across land owned by the Water District.  While the majority of transfer between Bell Marsh Reservoir and Boulter Pond takes place in a pipe, the final 1,000 feet of flow is overland.  The overland flow occurs across private property.  While the overland flow crosses remote, undeveloped land, private ownership means that future development could encroach on this area.

Water quality data collected at Boulter Pond in 1977 included total phosphorus and secchi depth.   These data reported total phosphorus at 8 ppb and secchi depth at 9 to 10 feet.  The Water District also collected routine turbidity and pH data on raw water from Boulter Pond.  Turbidity is typically near 2 NTUs and pH is typically near 6.  No water quality data are available from PEARL or the Water District for Folly or Middle Ponds or the Bell Marsh Reservoir.

Lakes and ponds normally experience an aging process known as eutrophication, which is caused by various natural and man-made influences.   Phosphorus from runoff, fertilizers, sewage and other sources is a primary factor affecting eutrophication since phosphorus nourishes plant and algae growth in the pond.   The amount of algae productivity in the pond is characterized in terms of three Trophic States: (1) Oligotrophic = low, (2) Mesotrophic = moderate, and (3) Eutrophic = high.  The Trophic State can be identified based on chlorophyll and phosphorus content and by secchi disk transparency data, which measures the clarity of the pond water.   Available raw water quality data, the PEARL database and the State lakes database indicate that Boulter Pond is Mesotrophic.  This indicates that the pond is moderately sensitive to land uses.

5.0   SWAP RANKING AND RECOMMENDATIONS

5.1   Ranking of Susceptibility

The SWAP assessment factors indicate that overall susceptibility of the water quality in Folly, Middle and Boulter Ponds and the Bell Marsh Reservoir is low.  This conclusion is based on the general conditions observed, including the density of development in the watersheds, the absence of activities that handle chemicals in the watersheds, and the prohibition of recreational uses of the water bodies.  Specific factors considered in assessing the overall risk are summarized in Table 3.

# TABLE  3

KITTERY WATER DISTRICT

SURFACE WATER ASSESSMENT

| Zone | Measure | Findings | Risk Level |
| --- | --- | --- | --- |
| Watershed | Ambient Water Quality Existing Conditions Future Development Overall | Class GPA, in full compliance. More than 99 percent of the land in the watersheds is undeveloped. Development pressure in the region is high. However 60 percent of the watershed is owned by KWD. Development on other land is restricted by zoning. | Low Low Low Low |
| Shoreland | Pond Classification Soils Activities Posing a Threat Potential for Future Threats Overall | Mesotrophic (Boulter), Others unclassified Erosion observed along Bell Marsh Road. Erodible soils common on ponds. Shoreline owned by KWD and undeveloped. Future shoreline development restricted by ownership and zoning. | Low-Moderate Moderate Low Low Low-Moderate |
| Intake | Raw Water Quality Ownership/Control Activities Posing a Threat Potential for Future Threats Overall | Recent pond quality is good. Extensive Water District ownership. No recreational or threatening uses. District ownership limits future development. | Low Low Low Low Low |
| Overall |  |  | Low |

5.2   Recommendations

While the overall ranking for the susceptibility of Kittery Water District’s supply ponds and reservoir is low, there are several additional actions that could be considered by the Water District and the Towns of York and South Berwick to provide added protection to source water quality.

- Planning and reconstruction of Bell Marsh Road should be done to provide maximum protection for the Reservoir.  Improvements might include a stormwater drainage system to include settling basins or other structures to control the quality of runoff into the reservoir.

- The Water District should work with the Town of South Berwick to raise awareness of the Bell March Reservoir watershed and increase zoning controls on future development in the watershed.

- Signs might be erected along Bell Marsh Road for public education to alert motorists when they enter and leave the watershed.

- The Water District may want to plan for future acquisition of land within the watershed.  Ownership is the most reliable means to prevent future development that may threaten the water supply.

- The Water District may consider collecting baseline water quality data to classify the trophic status of all of its sources.  Period sampling could also be instituted to monitor possible changes in the future.

#### APPENDIX A

SWAP ASSESSMENT GUIDELINES

# MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

BOULTER, FOLLY & MIDDLE PONDS

AND BELL MARSH RESERVOIR WATERSHEDS

# APPENDIX A

# ASSESSMENT GUIDELINES

| Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Physical Characteristics of the Watershed Watershed boundary, area Area of water body Tributaries Watershed topography Wetlands Water body depth Sand and gravel aquifers Soil types (erodibility and slope) | smaller smaller few low many deep few low | >> >> >> moderate >> >> some moderate | larger larger many steep, rugged few shallow many high |
| Raw Water Quality Secchi Disk Transparency Turbidity pH Phosphorus Pesticides and herbicides Color Total Coliform and E Coli Dissolved Oxygen Temperature VOCs Chlorophyll-a Total Organic Carbon | > 8 M < 1 NTU 6.5-8.5 < 10 ppb ND < 5 > 29/100 ml >7 mg/L ambient absent < 2 ppb < 4 mg/L | 4-8 M < 5 NTU N/A < 20 ppb N/A 5-15 occasional 5-7 mg/L N/A 2-6 ppb 4-8 mg/L | < 4 M > 5 NTU < 6.5 or > 8.5 > 30 ppb detected > 15 > 142/100 ml < 5 mg/L > 10o C ambient detected > 6 ppb > 8 mg/L |
| Potential Contamination Source Inventory UST/AST leak sites Municipal/Comm/Special Landfill Compost sites Demolition debris sites Ash/septage storage/utilization Superfund site Sand/salt storage sites Petroleum spill sites Hazardous waste sites Wastewater treatment facilities Food residuals utilization sites Uncontrolled hazardous sites Tank farms Industrial complexes | Absent from watershed or actively monitored and controlled or fully remediated. | Present in watershed, some operational deficiencies and/or remediation are underway. | Present and unmonitored and/or unremediated. |

| ### Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Potential Contamination Source Inventory Non-point pollution sources Transfer stations Sludge utilization sites Automobile graveyards Engineered subsurface systems Woodyards Underground injection wells Surface impoundments Mining/mineral processing sites Overboard discharges to rivers Active UST sites Roadways Railroads Utility Rights-of-way Boat launches Commercial farms CAFOs Commercial forest operations Private septic systems Home heating oil tanks Airports Bathing beaches Combined sewage overflows | If present, at low density, properly installed and operated. In compliance with applicable local, state and federal regulations. | >>>> | High density, little evidence of active management, documented incidence of problems and failures. |
| Land Uses and Zoning Residential density Percent of watershed Owned by PWS Public lands or conservation Forested With protective zoning Agricultural Impervious cover Industrial/commercial Waterbody uses: Restricted areas Ice fishing Boating Swimming Recreational vehicles | > 2 Ac/lot total > 80% for all protected uses < 20 % total < 20 % total < 20 % total large, posted limited to absent | 1-2 Ac/lot intake and shoreland protected, others developed 20-30% total 20-30% total 20-30% total intake zone moderate, well-managed | < 1 Ac/lot critical areas (intake, shoreland) unprotected or developed > 30% total of 3 > 30% total of 3 > 30% total of 3 small to none prevalent, uncontrolled |
| Other Analyses Trophic State Index Vulnerability Index Build-out Analysis | < 25 low fully developed | 25-60 moderate >>> | > 60 high high potential for development |